LONTIUM SEMICONDUCTOR CORPORATION

LT1501  Series

LT8912B Linux Driver Software

Debugging Guide

目  录

一  Introduction	3

二  Driver File Introduction	4

三  Add To The Kernel Method	7

四  LT8912B Software Configuration	9

五  Other Debugging Methods	11

# 一  Introduction

- The LT8912B does not have a built-in MCU, and requires an external MCU or master SOC to read and write registers to the chip through the I2C to work normally.

- The chip register address is 16 bits (the actual read and write operation is based on 8-bit address, the read and write timing can refer to 8-bit address), in which the high 8 bits are the Bank address, and the low 8 bits are the offset address in the Bank.

- Example,the I2C address  is 8 bits 0x90 on the MCU. When used on the SOC, it needs to move one bit to the right，and take the higher 7 bits to 0x48.

- The LT8912B chip has three I2C addresses and needs to switch three I2C addresses during initialization.(0x48,0x49,0x4a)

# 二  Driver File Introduction

- ① Driver file layout：

The main documents are described as follows:

- doc:            Directory for storing the introduction document

- include:        Store all header files of the driver.

- src:            Store all *.c files of the driver.

- Kconfig :       The configuration file is used to build the driver.

- Makefile:       The configuration file is used to build the driver.

- LontiumDrv.c:   Linux driver entry file. view the driver logic from this file

- LT8912B_Main.c:  The chip configuration entry  file is where the chip configuration logic begins.

- Other files:    Provides functions that are called during register configuration.

- ② LontiumDrv.c source code introduction：

- "lontium,lt8912b":  Key characters that match the device tree.

- chip_probe ：    Once the driver matches the device tree when insmod, chip_probe is executed.

- chip_pm_ops：    Register the sleep function -chip_suspend and wake function -chip_resume, if the platform's power cannot use driver control, also can use  control the reset pin.

- device tree:  Have three I2C address -  0x48、0x49 and 0x4a.

- ③ chip_probe source code introduction：

- ret = chip_parse_dts(lt8912b->dev):  Parses the GPIO resources configured in the device tree.

- kthread_obj = kthread_run(LT8912B_Main, NULL, "LT8912B_kthread"): Create a thread, call LT8912B_Main function, LT8912B_Main function will initialize the chip.

- 三  Add To The Kernel Method

- Take adding the source code to the drivers/video directory of the kernel as an example. Create the LT8912B folder in the video directory:

Copy the provided driver source file into the LT8912B directory and configure the driver/video/Makefile and driver/video/Kconfig files that already exist in the video directory:

  - driver/video/Makefile:

  - driver/video/Kconfig:

- Go to the LT8912B directory and add the driver/video/LT8912B/Makefile and driver/video/LT8912B/

- Kconfig, the source code for these two files is provided, and the configuration content is as follows:

  - driver/video/LT8912B/Makefile:

  - driver/video/LT8912B/Kconfig:

③ Execute the make menuconfig command in the kernel directory, and then configure the added driver to participate in build at build time.At the red line below:

- Select M to compile into a separate ko, which requires manual insmod.

- Select y to be automatically loaded when the kernel starts.

- Select n not to participate in compilation.

- Click Save and Exit to add the configuration to the configuration file  .config, and then  build.

[NOTE]： Also Can set CONFIG_LT8912B to a fixed value in xxxx_defconfig for your platform, so that you don't need make menuconfig.

# 四  LT8912B Software Configuration

- 五  Other Debugging Methods

In the early debugging phase, instead of porting code to the kernel directory, driver can be build separately using the following Makefile:

Run the make all command to build  the lt8912b.ko driver.

insmod lt8912b.ko  for init chip.